- LED Blink :

#define LED_PIN 2

void setup(){

pinMode(LED_PIN,OUTPUT);

}

void loop(){

digitalWrite(LED_PIN,HIGH);

delay(100);

digitalWrite(LED_PIN,LOW);

delay(100);

}

- Ultrasonic Sensor:

#define TRIG 5

#define ECHO 18

long duration;

int distance;

void setup()

{

pinMode(TRIG, OUTPUT);

pinMode(ECHO, INPUT);

Serial.begin(115200);

}

void loop()

{

digitalWrite(TRIG, LOW);

delayMicroseconds(2);

digitalWrite(TRIG, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG, LOW);

duration = pulseIn(ECHO, HIGH);

distance = duration * 0.034 / 2;

Serial.println(distance);

delay(1000);

}

- LDR:

#define LDR 34

#define LED 4

int value;

void setup()

{

pinMode(LED, OUTPUT);

Serial.begin(115200);

}

void loop()

{

value = analogRead(LDR);

Serial.println(value);

if(value < 100)

{

digitalWrite(LED, HIGH);

}

else

{

digitalWrite(LED, LOW);

}

delay(500);

}

- DC Motor:

#define ENA 14

#define IN1 27

#define IN2 26

void setup()

{

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

ledcAttach(ENA, 5000, 8);

}

void loop()

{

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

analogWrite(ENA, 80);

delay(2000);

analogWrite(ENA,150);

delay(2000);

analogWrite(ENA, 255);

delay(2000);

}

- DHT11:

#include <WiFi.h>

#include "ThingSpeak.h"

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

const char* ssid = "YOUR HOTSPOT NAME ";

const char* password = "PASSWORD";

WiFiClient client;

unsigned long channelNumber = 3376193;

const char* writeAPIKey = "Your API Key";

void setup()

{

Serial.begin(115200);

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

Serial.println("WiFi Connected");

ThingSpeak.begin(client);

dht.begin();

}

void loop()

{

float t = dht.readTemperature();

float h = dht.readHumidity();

ThingSpeak.setField(1, t);

ThingSpeak.setField(2, h);

Serial.println(t);

Serial.println(h);

ThingSpeak.writeFields(channelNumber, writeAPIKey);

delay(15000);

}

- Raspberry Pi LED Blink Code:

import RPi.GPIO as GPIO

import time

LED = 11

GPIO.setmode(GPIO.BOARD)

GPIO.setup(LED, GPIO.OUT)

while True:

GPIO.output(LED, 1)

time.sleep(1)

GPIO.output(LED, 0)

time.sleep(1)

- Rasberry Pi DHT11 code :

import RPi.GPIO as GPIO

import dht11

import time

import datetime

import requests

WRITE_API_KEY = "YOUR API KEY "

THINGSPEAK_URL = "https://api.thingspeak.com/update"

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

instance = dht11.DHT11(pin=14)

try:

while True:

result = instance.read()

if result.is_valid():

temperature = result.temperature

humidity = result.humidity

print("Temperature:", temperature)

print("Humidity:", humidity)

payload = {

'api_key': WRITE_API_KEY,

'field1': temperature,

'field2': humidity

}

response = requests.get(THINGSPEAK_URL, params=payload)

if response.status_code == 200:

print("Data sent successfully")

time.sleep(15)

except KeyboardInterrupt:

GPIO.cleanup()